Design and Implementation of Single Precision FPU in RISC-V Processor

Final year project report submitted in partial fulfilment of requirement for degree of Bachelors of Science in Electrical Engineering

| Arslan Ahmed | NIM-BSEE-2021-09 |
| --- | --- |
| Nasir Abbas | NIM-BSEE-2021-35 |

Dr. Naureen Shaukat

Department of Electrical Engineering

Namal University, Mianwali

2025

DECLARATION

The project report titled “Design and Implementation of Single Precision FPU in RISC-V Processor” is submitted in partial fulfilment of the degree of Bachelors of Science in Electrical Engineering, to the Department of Electrical Engineering at Namal University, Mianwali.

It is declared that this is an original work done by the team members listed below, under the guidance of our supervisor “Dr. Naureen Shaukat”. No part of this project and its report is plagiarised from anywhere, and any help taken from previous work is cited properly.

No part of the work reported here is submitted in fulfilment of requirement for any other degree/ qualification in any institute of learning.

Team Members						Signatures

Arslan Ahmed			NIM-BSEE-2021-09				______________

Nasir Abbas			NIM-BSEE-2021-35				______________

Supervisor

Dr. Naureen Shaukat				Signatures with date ____________________

Table of Contents

Abstract	v

Introduction	1

Literature Review	2

Methodology	3

3.1	Design Process	3

3.2	Mathematical Model	3

3.3	Algorithm (if any)	3

3.4	Block Diagram	3

3.5	Implementation Constraints	3

Testing	4

Results	5

Discussion	6

Conclusion	7

Future Work	8

Reflections on Learning	9

References	10

APPENDICES	11

List of Figures

List of Tables

ACKNOWLEDGMENTS

This project's success was not solely due to our efforts, but also the contributions of many other individuals who played crucial roles in its completion. We would like to express our sincere appreciation to our professors, who have been instrumental in shaping our project's success through their valuable guidance and encouragement.

We are truly appreciative to our Respected supervisor, Dr. Naureen Shaukat, for all of her help, encouragement, and support during this project. Her expertise and views were crucial in forming this work and advancing us. We really appreciate the ways in which she has assisted us, such as with weekly base progress reports, developing complicated concepts, offering constructive feedback, and assisting with obstacles.

We also want to express our sincere gratitude to one another for our outstanding teamwork, commitment, and tenacity during these endeavors. By cooperating, we overcame obstacles and accomplished our goals.

We also express our gratitude to our friends and colleagues for their moral support and useful advice. Finally, we would especially want to thank our families for their unwavering support and tolerance during this difficult project phase.

# Abstract

RISC-V, an open-source instruction set architecture, has characteristics of high performance, modularity, simplicity and easy expansion therefore numerous open- source cores have been employed in academia and commercial projects within a few years. Floating-Point Units (FPUs) are essential for the calculations of floating point numbers. The project aims to design a Single Precision Floating-Point Unit (FPU) and its integration into RISC-V based processor PicoRV32 which lacks dedicated FPU. FPU is suitable for performing arithmetic and logical operations on floating point numbers. The FPU is designed according to the IEEE-754-2008 standard. To support FPU, different modules like a register file are updated to store floating point numbers, and the decoder is updated to accommodate the floating-point instructions. FPU is integrated as a coprocessor in the PicoRV32 processor core and implemented on Field Programmable Gate Array (FPGA) for hardware verification. The results confirm the successful integration of the designed module in the core. Tests are conducted to compare the core’s performance without FPU and with FPU, ensuring that integrated FPU significantly enhances the core’s performance of handling floating point numbers.

Chapter 1

- Introduction

This section covers key preliminary concepts essential for understanding the upcoming project details.

- RISC-V Processor

- Background

- PicoRV32

- Floating Point Unit

- SDG’s Goals

- Purpose

- RISC-V Processor

The processor is the brain of a computer that helps to execute different instructions and perform various calculations. With the advancement in technology, the number of processors is increasing day by day so there are numerous processors available in the market. Companies are working on them to make them according to the needs of the customers and their use. Due to the dynamic nature of the market, the count is not possible. On a regular basis, they are created, modified, and changed to meet better performance and power requirements. RISC-V is a new kind of royalty-free architecture committed by the engineers at University of California, Berkeley. RISC-V is open standard instruction set architecture (ISA) based on the established reduced instruction set principle. Now it is gaining popularity in processor design under open-source licenses in specific applications such as Storage devices, Edge Computing and AI applications.

- Background

RISC-V based processors have changed the way to design processors. In the beginning, the processors were designed using the CISC approach, which was complex for new designers. However, this ISA is open source and allows a wide range of customization options to help computer architects design a processor according to their need [1]. The best thing about RISC-V processors is that they are simple, flexible, open source, and modular. Due its open source, modularity, scalability and simplicity billions of cores have been built by using its ISA. Developers optimize their designs according to their application requirements. This finds applications in both academic and commercial designs because it is free to use.

- Purpose

The main objective of the project is to design a single-precision Floating-Point Unit (FPU) for the PicoRV32 core that will be in accordance with IEEE 754-standard. This FPU will be able to perform accurate floating-point calculations, including many operations such as addition, subtraction, multiplication, and comparisons between numbers. The implementation of FPU integrated core on a Field Programmable Gate Array (FPGA) will enable core testing and validation under real-world conditions, ensuring its reliability and efficiency. 1 Ultimately, this research aims to overcome the constraints of lightweight RISC-V cores through the design of an efficient and resource-conscious single-precision FPU. The project’s success will be demonstrated through rigorous hardware testing and its potential applications, offering valuable insights into academic and technological development.

- PicoRV32

PicoRV32 is a RISC-V core developed by Clifford Wolf[1] and open-sourced for open contributions. PicoRV32 is a CPU core that implements the RISC-V RV32IMC Instruction Set, Integer (I), Multiply/Divide (M) and Compressed (C) extensions. It can be configured as RV32E, RV32I, RV32IC, RV32IM, or RV32IMC core, and optionally contains a built-in interrupt controller. It supports the optional Pico Co-Processor Interface (PCPI) that can be used to implement non-branching instructions in an external coprocessor. By implementations of PCPI any Standard Extension instructions can be integrated in PicoRV32.

- Floating Point Unit

Modern computing systems depend significantly on floating-point arithmetic to process real numbers accurately and with a wide dynamic range. This arithmetic capability is vital for numerous applications, such as scientific studies, digital signal processing and engineering simulations. The IEEE 754 standard for floating-point arithmetic provides accurate processing of floating-point integers [1]. RISC-V architecture has different core processors, the processor cores like PicoRV32, lack a dedicated Floating-Point Unit (FPU), which restricts their performance in high precision tasks. The small cores like the PicoRV32 are designed to minimize hardware complexity and energy consumption, which can compromise their computational strength. But a problem with this is they lack FPUs. This problem can be resolved by introducing a single-precision FPU that adheres with IEEE 754-standard, which will significantly improve PicoRV32’s ability to meet advanced computational requirements. In the integration process, there are some fundamental changes to the PicoRV32, including modifications to its register files, decoder unit and control logic to handle floating-point data and instructions. This will not only improve the core’s computational capabilities but also support the goals of open-source hardware innovation.

- SDG’s Goals

The Sustainable Development Goals (SDGs) are 17 global goals adopted by all United Nations Member States in 2015 as part of the 2030 Agenda for Sustainable Development. These goals aim to address major global challenges such as poverty, inequality, climate change, environmental degradation, peace, and justice. Each goal is broad, measurable, and has specific targets and indicators to track progress.

### SDG Goal 9: Industry, Innovation, and Infrastructure

Sustainable Development Goal 9 (SDG 09) includes building resilient infrastructure, promoting inclusive and sustainable industrialization, and fostering innovation. Moreover, our work coincides with SDG 09 by fostering innovation. By equipping PicoRV32 with enhanced floating-point features, the project contributes to developments in embedded systems, scientific research, and computationally intensive fields.

#### Objectives:

- Develop quality, reliable, sustainable, and resilient infrastructure

- Promote inclusive and sustainable industrialization

- Enhance scientific research, upgrade technological capabilities, and foster innovation

#### Project relation with SDG 09

Our project involves RISC-V processor enhancement, FPU design, embedded systems, and RISC-V development so it relate to goal 09

- Innovation & Research

- Industry Readiness

- Infrastructure Development

out a brief overview of the project and the background information required to understand different parts of it. Context for the project and reason for its selection should be provided in this section. Use as simplified language as you can (without technical terms and details) so that anyone with some basic knowledge is able to understand.

Mapping of the project goals with the United Nation’s Sustainable Development goals, as well as the effect of the project on environment and sustainability in general should be provided.

It should include the problem statement, context, aims and objectives of the project, and some overview of the solution/ criteria for completion. Provide a summary of the report structure and a brief outline of the chapters to come.

Chapter 2

# Literature Review

The development and optimization of Floating-Point Units (FPUs) have been extensively researched due to their critical role in modern processors. FPUs enable processors to perform floating-point arithmetic efficiently, a fundamental requirement for fields like scientific computation, real-time graphics, and advanced system processing. The IEEE 754 standard, established in 1985, has been instrumental in defining consistent and accurate methods for floating-point representation and operations, ensuring uniformity across hardware and software platforms.

3.1 FPUs in Processor Architectures

The integration of FPUs into processors significantly enhances their computational capabilities. For instance, Murali Krishna et al. (2013) demonstrated the advantages of incorporating FPUs for handling both simple and complex arithmetic operations [2]. Their design emphasized reducing hardware complexity while improving resource utilization. Similarly, Harshita Nair et al. (2020) presented an efficient implementation of single-precision floating-point operations on FPGA platforms [3]. By employing techniques such as pipelining and truncation, their work achieved notable improvements in speed and area efficiency, adhering to IEEE 754 guidelines. Despite advancements in FPU designs, lightweight cores like PicoRV32 often omit FPUs to maintain simplicity and energy efficiency. This exclusion limits their application in tasks requiring high precision and computational power. Addressing this challenge, Saghir (2010) highlighted the potential of application-specific instruction-set architectures in balancing performance with resource constraints. This perspective aligns with the objectives of this research, which seeks to integrate an efficient FPU into the PicoRV32 core.

3.2 Challenges in FPU Design

The design of FPUs poses several challenges, including hardware complexity, compliance with IEEE 754 standards, and optimization for power, area, and latency. Innovative architecture is being designed to address these challenges, emphasizing reduced lookup table usage and improved precision without excessive resource overhead. Harshita Nair et al. (2020) further explored trade-offs between latency and area, showcasing designs that achieve compactness and efficiency [3]. 2 For the PicoRV32 core, integrating an FPU requires significant modifications to its microarchitecture. These changes include updating register files to support floating-point data and adapting control logic to handle floating-point instructions. Louca et al. (1996) discussed similar architectural adjustments necessary for enabling floating-point operations in lightweight processors, providing valuable insights for this project.

3.3 Hardware Validation and FPGA Implementation

The practical validation of FPU designs often involves implementation on FPGA platforms. FPGAs provide a flexible and efficient means to test and refine hardware architectures. Harshita Nair et al. (2020) demonstrated the use of Xilinx FPGAs to validate single-precision floating point designs, highlighting the importance of efficient mapping and synthesis techniques [3]. The Xilinx Artix-7 FPGA, selected for this project, offers an optimal balance between resource availability and computational capability. Testing the designed FPU on an FPGA ensures its functionality and performance under real world conditions. Patterson and Hennessy (2020) emphasized the significance of hardware validation in processor design, noting its role in identifying and addressing architectural bottlenecks [5]. By leveraging FPGA testing, this research aims to refine the integrated PicoRV32 core, ensuring it meets its intended objectives of enhanced computational precision and efficiency.

Chapter 3

- Methodology

- Thoroughly study the RISC-V, PicoRv32 core

PicoRV32 is a CPU core that implements the RISC-V RV32IMC Instruction Set. It can be configured as RV32E, RV32I, RV32IC, RV32IM, or RV32IMC core, and optionally contains a built-in interrupt controller.

- Features and Typical Applications

- Small (750-2000 LUTs in 7-Series Xilinx Architecture)

- High fmax (250-450 MHz on 7-Series Xilinx FPGAs)

- Selectable native memory interface or AXI4-Lite master

- Optional IRQ support (using a simple custom ISA)

- Optional Co-Processor Interface

This CPU is meant to be used as auxiliary processor in FPGA designs and ASICs. Due to its high fmax it can be integrated in most existing designs without crossing clock domains. When operated on a lower frequency, it will have a lot of timing slack and thus can be added to a design without compromising timing closure. Furthermore it is possible to choose between a dual-port and a single-port register file implementation. The former provides better performance while the latter results in a smaller core. The core exists in three variations:

picorv32,

picorv32_axi

picorv32_wb.

The first provides a simple native memory interface that is easy to use in simple environments. picorv32_axi provides an AXI-4 Lite Master interface that can easily be integrated with existing systems that are already using the AXI standard. picorv32_wb provides a Wishbone master interface.

A separate core picorv32_axi_adapter is provided to bridge between the native memory interface and AXI4. This core can be used to create custom cores that include one or more PicoRV32 cores together with local RAM, ROM, and memory-mapped peripherals, communicating with each other using the native interface, and communicating with the outside world via AXI4.

The optional IRQ feature can be used to react to events from the outside, implement fault handlers, or catch instructions from a larger ISA and emulate them in software.

The optional Pico Co-Processor Interface (PCPI) can be used to implement non-branching instructions in an external coprocessor. Implementations of PCPI cores that implement the M Standard Extension instructions MUL[H[SU|U]] and DIV[U]/REM[U] are included in this package.

Below are the different test for picorv32

- Cycles per Instruction Performance

Unless stated otherwise, the following numbers apply to a PicoRV32 with ENABLE_REGS_DUALPORT active and connected to a memory that can accommodate requests within one clock cycle.

The average Cycles per Instruction (CPI) is approximately 4, depending on the mix of instructions in the code. The CPI numbers for the individual instructions can be found in the table below. The column "CPI (SP)" contains the CPI numbers for a core built without ENABLE_REGS_DUALPORT.

When ENABLE_MUL is activated, then a MUL instruction will execute in 40 cycles and a MULH[SU|U] instruction will execute in 72 cycles. When ENABLE_DIV is activated, then a DIV[U]/REM[U] instruction will execute in 40 cycles. When BARREL_SHIFTER is activated, a shift operation takes as long as any other ALU operation.

The following dhrystone benchmark results are for a core with enabled ENABLE_FAST_MUL, ENABLE_DIV, and BARREL_SHIFTER options. Dhrystone benchmark results: 0.516 DMIPS/MHz (908 Dhrystones/Second/MHz) For the Dhrystone benchmark the average CPI is 4.100. Without using the look-ahead memory interface , these results drop to 0.305 DMIPS/MHz and 5.232 CPI.

- Pico Co-Processor Interface (PCPI)

The Pico Co-Processor Interface (PCPI) can be used to implement non-branching instructions in external cores:

When an unsupported instruction is encountered and the PCPI feature is activated (see ENABLE_PCPI above), then pcpi_valid is asserted, the instruction word itself is output on pcpi_insn, the rs1 and rs2 fields are decoded and the values in those registers are output on pcpi_rs1 and pcpi_rs2. An external PCPI core can then decode the instruction, execute it, and assert pcpi_ready when execution of the instruction is finished. Optionally a result value can be written to pcpi_rd and pcpi_wr asserted. The PicoRV32 core will then decode the rd field of the instruction and write the value from pcpi_rd to the respective register. When no external PCPI core acknowledges the instruction within 16 clock cycles, then an illegal instruction exception is raised and the respective interrupt handler is called. A PCPI core that needs more than a couple of cycles to execute an instruction, should assert pcpi_wait as soon as the instruction has been decoded successfully and keep it asserted until it asserts pcpi_ready. This will prevent the PicoRV32 core from raising an illegal instruction exception.

- Building a pure RV32I Toolchain

Run the following commands to build the complete toolchain:

- make download-tools

- make -j$(nproc) build-tools

The default settings in the riscv-tools build scripts will build a compiler, assembler and linker that can target any RISC-V ISA, but the libraries are built for RV32G and RV64G targets.

Follow the instructions below to build a complete toolchain (including libraries) that target a pure RV32I CPU.

Tjhe following commands will build the RISC-V GNU toolchain and libraries for a pure RV32I target, and install it in /opt/riscv32i:

# Ubuntu packages needed:

sudo apt-get install autoconf automake autotools-dev curl libmpc-dev \

libmpfr-dev libgmp-dev gawk build-essential bison flex texinfo \

gperf libtool patchutils bc zlib1g-dev git libexpat1-dev

sudo mkdir /opt/riscv32i

sudo chown $USER /opt/riscv32i

Git clone https://github.com/riscv/riscv-gnu-toolchain riscv-gnu-toolchain-rv32i

cd riscv-gnu-toolchain-rv32i

git checkout 411d134

git submodule update --init --recursive

mkdir build; cd build

../configure --with-arch=rv32i --prefix=/opt/riscv32i

make -j$(nproc)

The commands will all be named using the prefix riscv32-unknown-elf-, which makes it easy to install them side-by-side with the regular riscv-tools (those are using the name prefix riscv64-unknown-elf- by default).

Alternatively you can simply use one of the following make targets from PicoRV32's Makefile to build a RV32I[M][C] toolchain. You still need to install all prerequisites, as described above. Then run any of the following commands in the PicoRV32 source directory:

| Command | Install Directory | ISA |
| --- | --- | --- |
| make -j$(nproc) build-riscv32i-tools | /opt/riscv32i/ | RV32I |
| make -j$(nproc) build-riscv32ic-tools | /opt/riscv32ic/ | RV32IC |
| make -j$(nproc) build-riscv32im-tools | /opt/riscv32im/ | RV32IM |
| make -j$(nproc) build-riscv32imc-tools | /opt/riscv32imc/ | RV32IMC |

Or simply run make -j$(nproc) build-tools to build and install all four tool chains.

By default calling any of those make targets will (re-)download the toolchain sources. Run make download-tools to download the sources to /var/cache/distfiles/ once in advance.

Note: These instructions are for git rev 411d134 (2018-02-14) of riscv-gnu-toolchain.

## Linking binaries with newlib for PicoRV32

The tool chains (see last section for install instructions) come with a version of the newlib C standard library.

Use the linker script firmware/riscv.ld for linking binaries against the newlib library. Using this linker script will create a binary that has its entry point at 0x10000. (The default linker script does not have a static entry point, thus a proper ELF loader would be needed that can determine the entry point at runtime while loading the program.)

Newlib comes with a few syscall stubs. You need to provide your own implementation of those syscalls and link your program with this implementation, overwriting the default stubs from newlib. See syscalls.c in scripts/cxxdemo/ for an example of how to do that.

It must include the following:

- Design Process

Explain the design process for the project in detail. This must include the design standards used (if any), and the evidence for best design practices followed.

- Mathematical Model

Support the project design with a mathematical model.

- Algorithm (if any)

Explain the algorithm for logic etc. used in the project.

- Block Diagram

Describe your system with the help of a clear and concise block diagram.

- Implementation Constraints

Provide information about the constraints that effected the project implementation, and the measures used as a solution as a result.

Chapter 4

# Testing

Include the various test cases used to check the working of the solution. Explain the testing methodologies used in reference to the core functionalities defined at the proposal stage.

This chapter may be merged into either the methodology or the results if desired, but test cases must be described.

Chapter 5

# Results

Show a complete assessment of the solution implemented through the various test cases described in the previous chapter. The results must be supported by figure, plots, comparison tables etc. to increase understanding of the reader. Results section should not include all test results if there are too many, but the major trends etc. must be clearly noted and should be observable through the supporting evidence.

The complete results (if too many) may be included as an appendix.

Chapter 6

# Discussion

Discuss the results in an objective manner. The discussion section should include a review of what you were trying to accomplish? (the original problem), what actually happened? (results), why does that matter? (implications), how does the project not achieve all the best case scenarios? (limitations), what you were not able to do and why? (failings and their reasons).

Discussion should not include any new methods or results not provided in the methodology or results section.

Chapter 7

# Conclusion

The conclusion should complement the introduction. Provide a brief summary of how the project addresses the problem specified in the introduction. Provide overview of the preceding sections, including the methods used to implement the solution, those used to evaluate its working and how the evaluation results show that the problem has been addressed.

Similar to discussion chapter, the conclusions should not provide any new information, not already stated in the relevant sections of the report.

Chapter 8

# Future Work

Provide an overview of recommendations, and in which direction the project could be taken in the future, giving some information about what resources would be required to further develop it in that direction.

Chapter 9

# Reflections on Learning

Reflect on the overall learning from the project. Clearly communicate the technical and non-technical skills that were developed as a result of the project. Explain how the project has helped in the personality development, and what could have been improved to achieve a better result.

# References

Provide a comprehensive list of references cited using the IEEE format.

# APPENDICES

<Attach any extra information (complete code or a link to it, all results etc.), references to any documents as appendices starting from next page>

### Integrating FPU in PicoRV32 as a Coprocessor (PCPI-FPU Integration)

To integrate the Floating Point Unit (FPU) into the PicoRV32 core as a coprocessor module, we will utilize PCPI (Pico Co-Processor Interface), which is already available in PicoRV32. This method allows the FPU to execute floating-point instructions while keeping the main core lightweight.

## Step-by-Step FPU Integration in PicoRV32

### 1️⃣ Understanding PCPI in PicoRV32

PicoRV32 provides a PCPI (Pico Co-Processor Interface), which allows connecting external processing units like FPU, custom accelerators, or other hardware extensions.

The PCPI interface consists of the following control signals:

- pcpi_valid: Indicates that an instruction requires coprocessor execution.

- pcpi_insn: The instruction being executed.

- pcpi_rs1: First operand (from register file).

- pcpi_rs2: Second operand (from register file).

- pcpi_wr: Coprocessor writes back the result.

- pcpi_rd: The computed result from the coprocessor.

- pcpi_ready: Signals that the coprocessor has finished execution.

### 2️⃣ Modify PicoRV32 to Recognize Floating-Point Instructions

To integrate the FPU, the instruction decoder in PicoRV32 needs to recognize floating-point instructions and send them to the FPU via the PCPI interface.

- Identify Floating-Point Instructions

  - In RISC-V, floating-point instructions have the opcode 1010011 (7'b1010011 in binary).

  - The funct3 field determines the specific operation (ADD, SUB, MUL, DIV, etc.).

- Modify the Instruction Decoder in PicoRV32

  - Locate the section in picorv32_core.v where pcpi_valid is assigned.

  - Add a condition to detect floating-point instructions (7'b1010011) and set pcpi_valid = 1.

  - Ensure pcpi_insn, pcpi_rs1, and pcpi_rs2 are correctly assigned.

### 3️⃣ Connect the PCPI-FPU Module

The PCPI-FPU module (pcpi_fpu) acts as a bridge between the PicoRV32 core and the FPU.

- Instantiate the pcpi_fpu Module in picorv32_core.v
Add the following inside the PicoRV32 core:

- pcpi_fpu fpu_unit (

- .clk(clk),

- .reset(reset),

- .pcpi_valid(pcpi_valid),

- .pcpi_insn(pcpi_insn),

- .pcpi_rs1(pcpi_rs1),

- .pcpi_rs2(pcpi_rs2),

- .pcpi_wr(pcpi_wr),

- .pcpi_rd(pcpi_rd),

- .pcpi_ready(pcpi_ready)

- );

- Modify the PCPI Logic in PicoRV32

  - Locate the section where pcpi_valid is assigned.

  - Add logic to set pcpi_valid = 1 when a floating-point instruction is detected:

- assign pcpi_valid = (instr_opcode == 7'b1010011); // Detect FPU instructions

- Ensure PCPI Signals Work Correctly

  - The pcpi_fpu module must:

    - Accept operands (pcpi_rs1, pcpi_rs2).

    - Process the floating-point operation.

    - Return the result (pcpi_rd).

    - Indicate completion (pcpi_ready = 1).

### 4️⃣ Modify the FPU to Work with PCPI

- The fpu.v module should accept PCPI signals and return results accordingly.

- Ensure opcode handling matches the instruction format.

- Example: Modify the FPU to handle pcpi_valid and pcpi_ready properly:

always @(posedge clk or posedge reset) begin

if (reset) begin

pcpi_ready <= 0;

pcpi_wr <= 0;

end else if (pcpi_valid) begin

pcpi_rd <= fpu_result;

pcpi_ready <= 1;

pcpi_wr <= 1;

end else begin

pcpi_ready <= 0;

pcpi_wr <= 0;

end

end

### 5️⃣ Verify FPU Integration

To ensure successful integration:
✅ Synthesize and simulate the design in ModelSim/QuestaSim.
✅ Test with floating-point instructions (fadd.s, fmul.s, fdiv.s, etc.).
✅ Check that the PCPI interface correctly routes instructions to the FPU.
✅ Ensure correctness using a RISC-V software simulator (e.g., Spike, QEMU).

Single Precision Floating Point Unit (FPU)

The given code is a hardware description for a Single Precision Floating Point Unit (FPU) based on the IEEE 754 standard, written in Verilog. The FPU supports addition, subtraction, multiplication, and division for single-precision floating-point numbers (32-bit format). Below is an explanation of the module and functionality.

# Level Module: fpu

## Purpose:

This is the main module that integrates arithmetic operations like addition, subtraction, multiplication, and division. It uses submodules for specific tasks and manages control logic to determine which operation to perform.

## Inputs and Outputs

input clk: Clock signal for synchronization.

input [31:0] A, B: Operands in IEEE 754 single-precision format.

input [1:0] opcode: Operation code defining the arithmetic operation (ADD, SUB, MUL, DIV).

output [31:0] O: Result of the operation in IEEE 754 format.

Intermediate Wires and Registers:

a_exponent, b_exponent (8-bit): Extracted exponent fields of A and B.

a_mantissa, b_mantissa (24-bit): Extracted mantissa fields of A and B. Implicit leading 1 is added.

o_sign, o_exponent, o_mantissa: Output components (sign, exponent, and mantissa).

Registers like adder_a_in, adder_b_in, etc., store operands for the respective operations.

## Opcode Decoding

The opcode is decoded to determine the operation type:

ADD: Addition operation (opcode = 00).

SUB: Subtraction operation (opcode = 01).

MUL: Multiplication operation (opcode = 11).

DIV: Division operation (opcode = 10).

## Submodule Instances:

Adder (adder A1): Performs floating-point addition.

Multiplier (multiplier M1): Performs floating-point multiplication.

Divider (divider D1): Uses a reciprocal approach for division.

Behavioral Logic (ALWAYS Block)

Based on opcode, the corresponding arithmetic operation is performed.

Special cases (like NaN, infinity, or zero) are handled explicitly.

The result is computed by invoking the respective arithmetic module, and the final IEEE 754 output is assembled using the o_sign, o_exponent, and o_mantissa.

## Arithmetic Modules

### Adder Module (adder)

Handles addition and subtraction of IEEE 754 floating-point numbers.

Step 1: Extract Components

Sign: Extracted from the most significant bit.

Exponent: Retrieved from bits [30:23] with special handling for denormalized numbers.

Mantissa: Appends an implicit leading 1 if the number is normalized.

Step 2: Align Exponents

The mantissa of the smaller number is right-shifted to match the larger exponent.

Step 3: Add/Subtract Mantissas

Depending on the signs, mantissas are added (same signs) or subtracted (opposite signs).

Step 4: Normalize Result

Leading zeros are removed from the mantissa, and the exponent is adjusted.

### Multiplier Module (multiplier)

Implements floating-point multiplication.

Step 1: Extract Components

Similar to the adder module.

Step 2: Compute Product

Mantissas are multiplied directly (48-bit product), and exponents are added with bias adjustment (a_exponent + b_exponent - 127).

Step 3: Normalize Product

If the leading bit of the product is 1, the product is shifted, and the exponent is incremented.

### Divider Module (divider)

Performs division using the formula

Result=A×(1/B).

Reciprocal Calculation: An auxiliary module computes the reciprocal of B.

Multiplication: The reciprocal is multiplied by A using the multiplier module.

## Normalization Modules

### Addition Normalizer (addition_normaliser)

Adjusts the mantissa and exponent after an addition or subtraction.

If leading bits of the mantissa are zero, the mantissa is shifted left, and the exponent is decreased.

### Multiplication Normalizer (multiplication_normaliser)

Adjusts the mantissa and exponent after multiplication.

Similar to the addition normalizer but operates on a 48-bit mantissa.

### Reciprocal Module (reciprocal)

Calculates the approximate reciprocal of a floating-point number.

This is used in the division module for faster computation.

# IEEE 754 Format Details

The 32-bit single-precision IEEE 754 format:

1-bit Sign (S): Determines the sign of the number.

8-bit Exponent (E): Encodes the exponent with a bias of 127.

23-bit Mantissa (M): Encodes the fractional part with an implicit leading 1 for normalized numbers.

## Special Cases

Handled in all operations:

### NaN (Not a Number):

Occurs when the exponent is 255 and the mantissa is non-zero.

### Infinity:

Represented by an exponent of 255 and a zero mantissa.

### Zero:

Special representation with zero exponent and mantissa.

# Installation of RISC-V GCC toolchain to run RISC-V PicoRV32 core

### 1. Update and Install Prerequisites

sudo apt update

sudo apt upgrade -y

sudo apt install build-essential gcc make git libgmp-dev libmpfr-dev libmpc-dev gawk bison flex texinfo gperf python3 python3-pip -y

### 2. Clone and Build the RISC-V GCC Toolchain

The RISC-V GNU Compiler Toolchain repository contains the necessary GCC for RISC-V:

# Clone the RISC-V GCC repository

git clone --recursive https://github.com/riscv-collab/riscv-gnu-toolchain

cd riscv-gnu-toolchain

# Build the GCC toolchain for RV32I (32-bit RISC-V ISA)

./configure --prefix=/opt/riscv --with-arch=rv32im --with-abi=ilp32

make -j$(nproc)

# Add the RISC-V GCC toolchain to your PATH

echo 'export PATH=/opt/riscv/bin:$PATH' >> ~/.bashrc

source ~/.bashrc

### 3. Set Up PicoRV32

Clone the PicoRV32 repository and test the setup:

git clone https://github.com/cliffordwolf/picorv32.git

cd picorv32

# Run the testbench to verify the build

make test

### 4. Compile Your Code for PicoRV32

Use the RISC-V GCC toolchain to compile code for the PicoRV32 core:

riscv32-unknown-elf-gcc -o your_program.elf -march=rv32im -mabi=ilp32 your_program.c

riscv32-unknown-elf-objcopy -O binary your_program.elf your_program.bin

### 5. Run the Code in PicoRV32 Testbench

Simulate the compiled program using the testbench provided in the PicoRV32 repository:

make firmware

make sim

This process should set up the GCC toolchain, build the PicoRV32 testbench, and allow you to simulate your programs. Let me know if you need additional steps!

## Step 1: Set Up the Environment

Make sure you have:

- Xilinx ISE Design Suite (e.g., ISE 14.7) – the last official tool for Spartan-6

- Your Spartan-6 development board (e.g., Nexys3, Papilio Pro)

- USB JTAG cable & drivers

- Optional: A UART interface or 7-seg/LEDs for output testing

## 🧾 Step 2: Create a New ISE Project

- Open ISE Design Suite

- Create New Project:

  - File > New Project

  - Name: fpu_test

  - Set the correct Spartan-6 device (e.g., xc6slx16-3-csg324)

- Add your fpu.v and all submodules (fp_add_sub.v, fp_multiply.v, etc.)

- Create a Top Module: This is your test wrapper that connects the FPU to FPGA pins (you'll write this in step 5).

## 🧪 Step 3: Simulate the FPU (Optional but Recommended)

- Create a testbench file (fpu_tb.v)

- Instantiate the FPU and drive it with test values

- Use the ISE simulator (ISim) to check output correctness

- Verify float operations behave as expected before moving to hardware

## 🏗️ Step 4: Write the Top-Level Wrapper Module

This will:

- Instantiate the FPU

- Provide input signals (e.g., via switches, buttons, or UART)

- Drive outputs to LEDs, 7-segment, or UART

Basic skeleton:

verilog

CopyEdit

module top (

input clk,

input rst,

input [3:0] opcode,

input [31:0] a,

input [31:0] b,

input load,

input store,

output [31:0] result

);

wire write_enable = 1'b1; // Always write in this test

wire [31:0] mem_data_in = 32'b0; // Placeholder

wire [4:0] rd = 5'b00000;        // Placeholder

wire [31:0] address = 32'b0;

fpu my_fpu (

.a(a),

.b(b),

.opcode(opcode),

.load(load),

.store(store),

.mem_data_in(mem_data_in),

.rd(rd),

.write_enable(write_enable),

.address(address),

.result(result),

.mem_data_out()

);

endmodule

You may need to modify it to match your I/O device layout (switches, LEDs, etc.)

## 🧱 Step 5: Assign Pins (UCF File)

- Create a .ucf file to map ports to physical FPGA pins.

Example for Nexys3:

ucf

CopyEdit

NET "clk" LOC = C9;     # 50MHz clock

NET "rst" LOC = C10;

NET "a[0]" LOC = ...;

...

NET "result[0]" LOC = ...; # Connect to LEDs or serial output

Use your board’s reference manual to get correct pin locations.

## 🛠️ Step 6: Implement Design

- Run:

  - Synthesize

  - Implement Design

  - Generate Programming File

- Fix any synthesis or implementation errors

## 🚀 Step 7: Program the FPGA

- Open iMPACT

- Connect to your FPGA board via JTAG

- Load the .bit file

- Program the device

## 🧪 Step 8: Verify on Hardware

- Use UART, LEDs, or a 7-segment to verify results.

- Input test values via switches or a UART script.

- Confirm expected output from the FPU matches theoretical calculations.

## 💡 Optional Enhancements:

- Add UART input/output interface for test flexibility

- Add a simple FSM to step through multiple test cases

- Log output to serial console for debugging

If you'd like, I can help you:

- Write a testbench

- Write the UART module or FSM for control

- Generate a sample UCF for your board

Let me know which board you're using!

4o

##### You said:

testbench should test all included instructions

2/2

###### ChatGPT said:

Perfect! Here's a Verilog testbench that covers all the included FPU instructions in your fpu.v file:

- fadd.s, fsub.s

- fmul.s, fdiv.s, fsqrt.s

- fmin.s, fmax.s

- fcvt.w.s, fcvt.wu.s, fcvt.s.w, fcvt.s.wu

- feq.s, flt.s, fle.s

We'll use #10 delays between tests and hardcode a few test vectors using IEEE-754 encoding for single-precision floats.

### ✅ Testbench: fpu_tb.v

verilog

CopyEdit

`timescale 1ns / 1ps

module fpu_tb;

reg [31:0] a, b;

reg [3:0] opcode;

reg load, store, write_enable;

reg [4:0] rd;

reg [31:0] address, mem_data_in;

wire [31:0] result;

wire [31:0] mem_data_out;

// Instantiate the FPU

fpu uut (

.a(a),

.b(b),

.opcode(opcode),

.load(load),

.store(store),

.mem_data_in(mem_data_in),

.rd(rd),

.write_enable(write_enable),

.address(address),

.result(result),

.mem_data_out(mem_data_out)

);

// Helper: real-to-hex conversion (for visual reference)

function [31:0] float_to_ieee;

input real val;

begin

float_to_ieee = $realtobits(val);

end

endfunction

initial begin

// Init

a = 0; b = 0; opcode = 0;

load = 0; store = 0; write_enable = 1;

rd = 0; address = 0; mem_data_in = 0;

// Test fadd.s: 1.5 + 2.5 = 4.0

a = float_to_ieee(1.5);

b = float_to_ieee(2.5);

opcode = 4'b0000;

#10;

// Test fsub.s: 5.5 - 2.0 = 3.5

a = float_to_ieee(5.5);

b = float_to_ieee(2.0);

opcode = 4'b0001;

#10;

// Test fmul.s: 3.0 * 2.0 = 6.0

a = float_to_ieee(3.0);

b = float_to_ieee(2.0);

opcode = 4'b0010;

#10;

// Test fdiv.s: 8.0 / 2.0 = 4.0

a = float_to_ieee(8.0);

b = float_to_ieee(2.0);

opcode = 4'b0011;

#10;

// Test fmin.s: min(1.2, 2.4) = 1.2

a = float_to_ieee(1.2);

b = float_to_ieee(2.4);

opcode = 4'b0100;

#10;

// Test fmax.s: max(1.2, 2.4) = 2.4

opcode = 4'b0101;

#10;

// Test fsqrt.s: sqrt(16.0) = 4.0

a = float_to_ieee(16.0);

opcode = 4'b0110;

#10;

// Test feq.s: 2.5 == 2.5 -> 1

a = float_to_ieee(2.5);

b = float_to_ieee(2.5);

opcode = 4'b1000;

#10;

// Test flt.s: 1.5 < 2.5 -> 1

a = float_to_ieee(1.5);

b = float_to_ieee(2.5);

opcode = 4'b1001;

#10;

// Test fle.s: 1.5 <= 1.5 -> 1

a = float_to_ieee(1.5);

b = float_to_ieee(1.5);

opcode = 4'b1010;

#10;

// Test fcvt.w.s: float 4.9 -> int 4

a = float_to_ieee(4.9);

opcode = 4'b1100;

#10;

// Test fcvt.wu.s: float 4.9 -> uint 4

opcode = 4'b1101;

#10;

// Test fcvt.s.w: int 5 -> float 5.0

a = 32'd5;

opcode = 4'b1110;

#10;

// Test fcvt.s.wu: uint 5 -> float 5.0

opcode = 4'b1111;

#10;

$finish;

end

endmodule

### 🔍 Next Steps

- Add fp_add_sub.v, fp_multiply.v, etc., to your simulation project.

- Run this testbench using ISim or another Verilog simulator.

- Observe result output in the waveform or terminal.

Let me know if you want:

- Real-to-float printing helpers ($bitstoreal)

- Console display of output in human-readable format

- Version for test automation using Python + UART

Want me to bundle this with a full simulation setup?

4o

##### You said:

datasheet_nexys3.pdf

PDF

This is my board datasheet read this and write UCF file so that i can test my designed fpu i will provide input in test bench and just output to be shown on the 7-segment display and leds

2/2

###### ChatGPT said:

Thanks for sharing the datasheet for the Nexys 3 board. Based on your request, I'll generate a UCF (User Constraints File) so that:

- You can input data via your testbench.

- The output of your FPU (Floating Point Unit) will be visible on the 7-segment displays and LEDs.

Assuming the following:

- You're driving the output of your FPU to 8 LEDs (LD0–LD7) and one or more 7-segment displays (CA–CG, DP, and AN0–AN3).

- You will manage segment multiplexing and decoding in your design.

- The clock and reset pins are optional or managed internally via simulation/testbench.

Here's a basic UCF example for Nexys 3:

ucf

CopyEdit

# Clock (if needed)

NET "clk" LOC = "E3" | IOSTANDARD = LVCMOS33;  # 100 MHz clock

# LEDs (LD0–LD7)

NET "led[0]" LOC = "T1" | IOSTANDARD = LVCMOS33;

NET "led[1]" LOC = "R1" | IOSTANDARD = LVCMOS33;

NET "led[2]" LOC = "P1" | IOSTANDARD = LVCMOS33;

NET "led[3]" LOC = "N1" | IOSTANDARD = LVCMOS33;

NET "led[4]" LOC = "T3" | IOSTANDARD = LVCMOS33;

NET "led[5]" LOC = "T2" | IOSTANDARD = LVCMOS33;

NET "led[6]" LOC = "R3" | IOSTANDARD = LVCMOS33;

NET "led[7]" LOC = "R2" | IOSTANDARD = LVCMOS33;

# 7-Segment Display segments (CA–CG, DP)

NET "seg[0]" LOC = "L14" | IOSTANDARD = LVCMOS33;  # CA

NET "seg[1]" LOC = "H12" | IOSTANDARD = LVCMOS33;  # CB

NET "seg[2]" LOC = "N14" | IOSTANDARD = LVCMOS33;  # CC

NET "seg[3]" LOC = "N11" | IOSTANDARD = LVCMOS33;  # CD

NET "seg[4]" LOC = "P12" | IOSTANDARD = LVCMOS33;  # CE

NET "seg[5]" LOC = "L13" | IOSTANDARD = LVCMOS33;  # CF

NET "seg[6]" LOC = "M12" | IOSTANDARD = LVCMOS33;  # CG

NET "seg[7]" LOC = "N13" | IOSTANDARD = LVCMOS33;  # DP

# 7-Segment Display anodes (AN0–AN3)

NET "an[0]" LOC = "M13" | IOSTANDARD = LVCMOS33;

NET "an[1]" LOC = "J12" | IOSTANDARD = LVCMOS33;

NET "an[2]" LOC = "F12" | IOSTANDARD = LVCMOS33;

NET "an[3]" LOC = "D14" | IOSTANDARD = LVCMOS33;





module fpu (

input [31:0] a, b,        // Floating-point inputs

input [31:0] mem_data_in, // Data from memory (for load instruction)

input [3:0] opcode,       // Operation selector (expanded to 4 bits to accommodate all operations)

input load,               // Load control signal

input store,              // Store control signal

input [31:0] address,     // Address for load/store operations

input [4:0] rd,           // Destination register index (0-31)

input write_enable,       // Write enable signal for registers

output reg [31:0] result, // Computed result

output reg [31:0] mem_data_out // Data to store in memory

);

// Floating-Point Register File

reg [31:0] f[31:0];  // 32 Floating-point registers f0-f31

reg [31:0] fcsr;     // Floating-Point Control and Status Register (fcsr)

wire [31:0] add_sub_result, mul_result, div_result, fmin_result, fmax_result, sqrt_result;

wire [31:0] cvt_w_result, cvt_s_result;

wire feq_result, flt_result, fle_result;

// Floating Point Operations

fp_add_sub adder_subtractor (.a(a), .b(b), .add_sub(opcode[0]), .result(add_sub_result));

fp_multiply multiplier (.a(a), .b(b), .result(mul_result));

fp_divide divider (.a(a), .b(b), .result(div_result));

fp_minmax min_max_unit (.a(a), .b(b), .op(opcode[0]), .result(fmin_result));  // MIN/MAX operation

fp_sqrt sqrt_unit (.a(a), .result(sqrt_result));  // SQRT operation

fp_compare compare_unit (.a(a), .b(b), .op(opcode[1:0]), .result_bit(feq_result), .result_lt(flt_result), .result_le(fle_result));

// New Conversion Operations

fp_to_int fp_to_int_unit (.a(a), .unsigned_flag(opcode[0]), .result(cvt_w_result));  // FCVT.W.S/FCVT.WU.S

int_to_fp int_to_fp_unit (.a(a), .unsigned_flag(opcode[0]), .result(cvt_s_result));  // FCVT.S.W/FCVT.S.WU

// Operation Selection

always @(*) begin

case (opcode)

4'b0000: result = add_sub_result;       // ADD

4'b0001: result = add_sub_result;       // SUB

4'b0010: result = mul_result;           // MUL

4'b0011: result = div_result;           // DIV

4'b0100: result = fmin_result;          // FMIN.S

4'b0101: result = fmax_result;          // FMAX.S

4'b0110: result = sqrt_result;          // FSQRT.S

4'b1000: result = {31'b0, feq_result};  // FEQ.S

4'b1001: result = {31'b0, flt_result};  // FLT.S

4'b1010: result = {31'b0, fle_result};  // FLE.S

4'b1100: result = cvt_w_result;         // FCVT.W.S (signed int conversion)

4'b1101: result = cvt_w_result;         // FCVT.WU.S (unsigned int conversion)

4'b1110: result = cvt_s_result;         // FCVT.S.W (signed int to float)

4'b1111: result = cvt_s_result;         // FCVT.S.WU (unsigned int to float)

default: result = 32'h00000000;

endcase

end

// Load/Store Operations

always @(*) begin

if (load) begin

if (address[1:0] != 2'b00) fcsr[3] = 1; // Misalignment exception

else result = mem_data_in;

end

if (store) begin

if (address[1:0] != 2'b00) fcsr[3] = 1; // Misalignment exception

else mem_data_out = a;

end

end

// Register Write Mechanism

always @(posedge write_enable) begin

f[rd] <= result;  // Write result to selected floating-point register

end

endmodule

module fpu (

input [31:0] a, b,        // Floating-point inputs

input [31:0] mem_data_in, // Data from memory (for load instruction)

input [3:0] opcode,       // Operation selector (expanded to 4 bits to accommodate new operations)

input load,               // Load control signal

input store,              // Store control signal

input [31:0] address,     // Address for load/store operations

input [4:0] rd,           // Destination register index (0-31)

input write_enable,       // Write enable signal for registers

output reg [31:0] result, // Computed result

output reg [31:0] mem_data_out // Data to store in memory

);

// Floating-Point Register File

reg [31:0] f[31:0];  // 32 Floating-point registers f0-f31

reg [31:0] fcsr;     // Floating-Point Control and Status Register (fcsr)

wire [31:0] add_sub_result, mul_result, div_result, fmin_result, fmax_result, sqrt_result;

wire feq_result, flt_result, fle_result;

// Floating Point Operations

fp_add_sub adder_subtractor (.a(a), .b(b), .add_sub(opcode[0]), .result(add_sub_result));

fp_multiply multiplier (.a(a), .b(b), .result(mul_result));

fp_divide divider (.a(a), .b(b), .result(div_result));

fp_minmax min_max_unit (.a(a), .b(b), .op(opcode[0]), .result(fmin_result));  // MIN/MAX operation

fp_sqrt sqrt_unit (.a(a), .result(sqrt_result));  // SQRT operation

fp_compare compare_unit (.a(a), .b(b), .op(opcode[1:0]), .result_bit(feq_result), .result_lt(flt_result), .result_le(fle_result));

// Operation Selection

always @(*) begin

case (opcode)

4'b0000: result = add_sub_result;       // ADD

4'b0001: result = add_sub_result;       // SUB

4'b0010: result = mul_result;           // MUL

4'b0011: result = div_result;           // DIV

4'b0100: result = fmin_result;          // FMIN.S

4'b0101: result = fmax_result;          // FMAX.S

4'b0110: result = sqrt_result;          // FSQRT.S

4'b1000: result = {31'b0, feq_result};  // FEQ.S

4'b1001: result = {31'b0, flt_result};  // FLT.S

4'b1010: result = {31'b0, fle_result};  // FLE.S

default: result = 32'h00000000;

endcase

end

// Load/Store Operations

always @(*) begin

if (load) begin

if (address[1:0] != 2'b00) fcsr[3] = 1; // Misalignment exception

else result = mem_data_in;

end

if (store) begin

if (address[1:0] != 2'b00) fcsr[3] = 1; // Misalignment exception

else mem_data_out = a;

end

end

// Register Write Mechanism

always @(posedge write_enable) begin

f[rd] <= result;  // Write result to selected floating-point register

end

endmodule

// Floating-Point Adder/Subtractor (Unchanged from original)

module fp_add_sub (

input [31:0] a, b,

input add_sub,  // 0 for addition, 1 for subtraction

output reg [31:0] result

);

wire sign_a = a[31];

wire sign_b = b[31] ^ add_sub;  // Flip sign for subtraction

wire [7:0] exp_a = a[30:23], exp_b = b[30:23];

wire [23:0] mant_a = {1'b1, a[22:0]};  // Implicit leading 1

wire [23:0] mant_b = {1'b1, b[22:0]};

wire exp_greater = (exp_a > exp_b);

wire [7:0] exp_diff = exp_greater ? (exp_a - exp_b) : (exp_b - exp_a);

wire [23:0] mant_a_shifted = exp_greater ? mant_a : (mant_a >> exp_diff);

wire [23:0] mant_b_shifted = exp_greater ? (mant_b >> exp_diff) : mant_b;

wire [23:0] mant_large = exp_greater ? mant_a : mant_b;

wire [23:0] mant_small = exp_greater ? mant_b_shifted : mant_a_shifted;

wire [7:0] exp_large = exp_greater ? exp_a : exp_b;

wire sign_large = exp_greater ? sign_a : sign_b;

wire sign_small = exp_greater ? sign_b : sign_a;

wire [24:0] mant_sum = (sign_large == sign_small) ?

(mant_large + mant_small) :

(mant_large - mant_small);

reg [7:0] exp_norm;

reg [23:0] mant_norm;

reg sign_result;

integer shift_count;

always @(*) begin

if (mant_sum[24]) begin

// Overflow occurred, shift right

exp_norm = exp_large + 1;

mant_norm = mant_sum[24:1];

end else begin

// Normalize by shifting left if necessary

mant_norm = mant_sum[23:0];

exp_norm = exp_large;

shift_count = 0;

while (mant_norm[23] == 0 && exp_norm > 0) begin

mant_norm = mant_norm << 1;

exp_norm = exp_norm - 1;

shift_count = shift_count + 1;

end

end

// Determine final sign

if ((sign_large != sign_small) && (mant_large < mant_small)) begin

sign_result = sign_small;  // Result takes the sign of the larger absolute value

mant_norm = (~mant_norm + 1);  // Two's complement for negative results

end else begin

sign_result = sign_large;

end

result = {sign_result, exp_norm, mant_norm[22:0]};

end

endmodule

// Floating-Point Multiplier (Unchanged from original)

module fp_multiply (

input [31:0] a, b,

output reg [31:0] result

);

wire sign_a = a[31], sign_b = b[31];

wire [7:0] exp_a = a[30:23], exp_b = b[30:23];

wire [23:0] mant_a = {1'b1, a[22:0]}, mant_b = {1'b1, b[22:0]};

wire sign_result = sign_a ^ sign_b;

wire [8:0] exp_result = exp_a + exp_b - 127;

wire [47:0] mant_result = mant_a * mant_b;

reg [22:0] mant_norm;

reg [7:0] exp_norm;

always @(*) begin

if (mant_result[47]) begin

mant_norm = mant_result[46:24];

exp_norm = exp_result + 1;

end else begin

mant_norm = mant_result[45:23];

exp_norm = exp_result;

end

result = {sign_result, exp_norm, mant_norm};

end

endmodule

// Floating-Point Divider (Unchanged from original)

module fp_divide (

input [31:0] a, b,

output reg [31:0] result

);

wire sign_a = a[31], sign_b = b[31];

wire [7:0] exp_a = a[30:23], exp_b = b[30:23];

wire [23:0] mant_a = {1'b1, a[22:0]}, mant_b = {1'b1, b[22:0]};

wire sign_result = sign_a ^ sign_b;

wire [8:0] exp_result = exp_a - exp_b + 127;

wire [47:0] mant_result = (mant_a << 23) / mant_b;

reg [22:0] mant_norm;

reg [7:0] exp_norm;

always @(*) begin

mant_norm = mant_result[22:0];

exp_norm = exp_result;

result = (b == 32'h00000000) ? 32'h7fc00000 : {sign_result, exp_norm, mant_norm};

end

endmodule

// Floating-Point Minimum and Maximum (FMIN.S, FMAX.S) - Unchanged from original

module fp_minmax (

input [31:0] a, b,

input op,  // 0 for FMIN, 1 for FMAX

output reg [31:0] result

);

function is_nan;

input [31:0] x;

is_nan = (x[30:23] == 8'hFF) && (x[22:0] != 0);

endfunction

always @(*) begin

if (is_nan(a) && is_nan(b)) begin

result = 32'h7FC00000; // Canonical NaN

end else if (is_nan(a)) begin

result = b;  // If only `a` is NaN, return `b`

end else if (is_nan(b)) begin

result = a;  // If only `b` is NaN, return `a`

end else if (op == 1) begin

if ((a[31] == 0) && (b[31] == 0)) begin

if(a[30:23] == b[30:23])begin

if(a[22:0] == b[22:0])begin

result = a;

end else if (a[22:0] > b[22:0]) begin

result = a;

end else if (a[22:0] < b[22:0]) begin

result = b;

end

end else if(a[30:23] > b[30:23])begin

result=a;

end else if(a[30:23] < b[30:23])begin

result=b;

end

end else if ((a[31] == 1) && (b[31] == 1))begin

if(a[30:23] == b[30:23])begin

if(a[22:0] == b[22:0])begin

result = a;

end else if (a[22:0] > b[22:0]) begin

result = b;

end else if (a[22:0] < b[22:0]) begin

result = a;

end

end else if(a[30:23] > b[30:23])begin

result=b;

end else if(a[30:23] < b[30:23])begin

result=a;

end

end else if ((a[31] == 1) && (b[31] == 0) ) begin

result=b;

end else if ((a[31] == 0) && (b[31] == 1) ) begin

result=a;

end else begin

result = 32'h7FC00000; // Canonical NaN

end

end else if (op == 0) begin

if ((a[31] == 0) && (b[31] == 0)) begin

if(a[30:23] == b[30:23])begin

if(a[22:0] == b[22:0])begin

result = a;

end else if (a[22:0] > b[22:0]) begin

result = b;

end else if (a[22:0] < b[22:0]) begin

result = a;

end

end else if(a[30:23] > b[30:23])begin

result=b;

end else if(a[30:23] < b[30:23])begin

result=a;

end

end else if ((a[31] == 1) && (b[31] == 1) ) begin

if(a[30:23] == b[30:23])begin

if(a[22:0] == b[22:0])begin

result = a;

end else if (a[22:0] > b[22:0]) begin

result = a;

end else if (a[22:0] < b[22:0]) begin

result = b;

end

end else if(a[30:23] > b[30:23])begin

result=a;

end else if(a[30:23] < b[30:23])begin

result=b;

end

end else if ((a[31] == 1) && (b[31] == 0) ) begin

result=a;

end else if ((a[31] == 0) && (b[31] == 1) ) begin

result=b;

end else begin

result = 32'h7FC00000; // Canonical NaN

end

end else begin

result = 32'h7FC00000;

end

end

endmodule

// NEW MODULE: Floating-Point Square Root (FSQRT.S)

module fp_sqrt (

input [31:0] a,

output reg [31:0] result

);

// Extract components

wire sign_a = a[31];

wire [7:0] exp_a = a[30:23];

wire [22:0] frac_a = a[22:0];

// Square root specific variables

reg [7:0] exp_result;

reg [22:0] frac_result;

reg [24:0] operand;  // For normalized mantissa with guard bits

reg [24:0] root;     // The computed square root value

reg [24:0] remainder;

reg [24:0] temp;

integer i;

// Handle special cases

wire is_zero = (exp_a == 0) && (frac_a == 0);

wire is_inf = (exp_a == 8'hFF) && (frac_a == 0);

wire is_nan = (exp_a == 8'hFF) && (frac_a != 0);

wire is_neg = sign_a && !is_zero;  // Negative and not zero

always @(*) begin

// Special cases handling

if (is_zero) begin

// Square root of zero is zero

result = a;  // Preserve sign of zero

end else if (is_inf && !sign_a) begin

// Square root of +infinity is +infinity

result = a;

end else if (is_nan || is_neg || (is_inf && sign_a)) begin

// Square root of NaN, negative number, or -infinity is NaN

result = 32'h7FC00000;  // canonical NaN

end else begin

// Normal computation for positive finite numbers

// Prepare the operand - normalize the mantissa

operand = {1'b1, frac_a, 1'b0};  // Include implicit 1 and guard bit

// Adjust exponent

if (exp_a[0]) begin  // Odd exponent

operand = {operand[23:0], 1'b0};  // Left shift by 1 for normalization

end

// Final exponent is half of the original (minus bias adjustment)

exp_result = ((exp_a - 127) >> 1) + 127;

// Non-restoring square root algorithm

root = 0;

remainder = 0;

for (i = 0; i < 24; i = i + 1) begin

remainder = {remainder[22:0], operand[24-i], operand[23-i]};

temp = {root, 1'b1};

if (remainder >= temp) begin

remainder = remainder - temp;

root = {root[22:0], 1'b1};

end else begin

root = {root[22:0], 1'b0};

end

end

// Round the result (simplified round-to-nearest)

if (remainder > 0) begin

root = root + 1;  // Round up if there's a remainder

end

// Normalize the result if needed

if (root[23]) begin

frac_result = root[22:0];

end else begin

frac_result = root[21:0];  // Shift left if leading bit is zero

exp_result = exp_result - 1;

end

// Assemble the final result

result = {1'b0, exp_result, frac_result};

end

end

endmodule

// NEW MODULE: Floating-Point Compare (FEQ.S, FLT.S, FLE.S)

module fp_compare (

input [31:0] a, b,

input [1:0] op,  // 00: FEQ, 01: FLT, 10: FLE

output reg result_bit, // General result bit

output reg result_lt,  // Less than result

output reg result_le   // Less than or equal result

);

// Check for NaN values

function is_nan;

input [31:0] x;

is_nan = (x[30:23] == 8'hFF) && (x[22:0] != 0);

endfunction

// Check for equality

wire equal = (a == b) || (is_zero(a) && is_zero(b));  // Consider both +0 and -0 as equal

// Check for zero value

function is_zero;

input [31:0] x;

is_zero = (x[30:23] == 0) && (x[22:0] == 0);  // Exponent and fraction are zero

endfunction

// Less than comparison logic

wire a_negative = a[31] && !(is_zero(a));  // Negative and not zero

wire b_negative = b[31] && !(is_zero(b));  // Negative and not zero

wire less_than = (a_negative && !b_negative) ||                                   // a negative, b positive

(a_negative && b_negative && (a[30:0] > b[30:0])) ||              // both negative, |a| > |b|

(!a_negative && !b_negative && (a[30:0] < b[30:0]));              // both positive, a < b

always @(*) begin

if (is_nan(a) || is_nan(b)) begin

// NaN values make all comparisons return false

result_bit = 0;

result_lt = 0;

result_le = 0;

end else begin

// Equal comparison

result_bit = equal;

// Less than comparison

result_lt = less_than;

// Less than or equal comparison

result_le = equal || less_than;

end

end

endmodule

// Floating-Point to Integer Conversion Module (FCVT.W.S, FCVT.WU.S)

module fp_to_int (

input [31:0] a,           // Floating-point input

input unsigned_flag,      // 0 for signed (FCVT.W.S), 1 for unsigned (FCVT.WU.S)

output reg [31:0] result  // Integer result

);

// Extract components

wire sign_a = a[31];

wire [7:0] exp_a = a[30:23];

wire [22:0] frac_a = a[22:0];

wire [23:0] mant_a = {1'b1, frac_a}; // Implicit leading 1

// Special cases

wire is_zero = (exp_a == 0) && (frac_a == 0);

wire is_inf = (exp_a == 8'hFF) && (frac_a == 0);

wire is_nan = (exp_a == 8'hFF) && (frac_a != 0);

// Variables for conversion

reg [31:0] int_value;

reg [7:0] shift_amount;

always @(*) begin

// Handle special cases

if (is_zero) begin

result = 32'h00000000;  // Zero maps to zero

end

else if (is_nan || is_inf) begin

if (unsigned_flag) begin

result = (sign_a && !is_nan) ? 32'h00000000 : 32'hFFFFFFFF;  // -Inf -> 0, +Inf/NaN -> max unsigned

else

result = (sign_a && !is_nan) ? 32'h80000000 : 32'h7FFFFFFF;  // -Inf -> min signed, +Inf/NaN -> max signed

end

end

else begin

// Calculate unbiased exponent

shift_amount = exp_a - 127;

if (shift_amount > 31) begin

// Overflow cases

if (unsigned_flag) begin

result = (sign_a) ? 32'h00000000 : 32'hFFFFFFFF;  // Negative -> 0, Positive -> max unsigned

end else begin

result = (sign_a) ? 32'h80000000 : 32'h7FFFFFFF;  // Negative -> min signed, Positive -> max signed

end

end

else if (shift_amount < 0) begin

// Fractional numbers between -1 and 1

result = 32'h00000000;  // Truncate to zero

end

else begin

// Normal conversion

if (shift_amount <= 23) begin

// Shift mantissa according to exponent value

int_value = mant_a >> (23 - shift_amount);

end else begin

// Need to shift left for large exponents

int_value = mant_a << (shift_amount - 23);

end

// Handle sign for signed integers

if (sign_a) begin

if (unsigned_flag)

result = 32'h00000000;  // Negative float to unsigned int is 0

else

result = (~int_value + 1);  // Two's complement for negative

end else begin

result = int_value;  // Positive value stays the same

end

end

end

end

endmodule

// Integer to Floating-Point Conversion Module (FCVT.S.W, FCVT.S.WU)

module int_to_fp (

input [31:0] a,           // Integer input

input unsigned_flag,      // 0 for signed (FCVT.S.W), 1 for unsigned (FCVT.S.WU)

output reg [31:0] result  // Floating-point result

);

reg sign_result;

reg [7:0] exp_result;

reg [22:0] frac_result;

reg [31:0] abs_value;

integer leading_zeros;

always @(*) begin

// Handle sign

if (unsigned_flag) begin

sign_result = 1'b0;  // Always positive for unsigned

abs_value = a;

end else begin

sign_result = a[31];  // Sign bit of signed integer

abs_value = sign_result ? (~a + 1) : a;  // Two's complement if negative

end

// Handle zero special case

if (abs_value == 0) begin

result = 32'h00000000;  // +0.0

end else begin

// Count leading zeros to normalize

leading_zeros = 0;

if (abs_value[31:16] == 0) begin leading_zeros = leading_zeros + 16; abs_value = abs_value << 16; end

if (abs_value[31:24] == 0) begin leading_zeros = leading_zeros + 8; abs_value = abs_value << 8; end

if (abs_value[31:28] == 0) begin leading_zeros = leading_zeros + 4; abs_value = abs_value << 4; end

if (abs_value[31:30] == 0) begin leading_zeros = leading_zeros + 2; abs_value = abs_value << 2; end

if (abs_value[31] == 0) begin leading_zeros = leading_zeros + 1; abs_value = abs_value << 1; end

// Calculate exponent (biased)

exp_result = 8'd127 + 8'd31 - leading_zeros;

// Calculate fraction (mantissa without implied 1)

frac_result = abs_value[30:8];

// Handle rounding (simplified round-to-nearest)

if (abs_value[7]) begin

// Check if we need to round up

if (frac_result == 23'h7FFFFF) begin

// If all fraction bits are 1, rounding up would cause overflow

frac_result = 0;

exp_result = exp_result + 1;

end else begin

frac_result = frac_result + 1;

end

end

// Construct result

result = {sign_result, exp_result, frac_result};

end

end

endmodule

`timescale 1ns / 1ps

module fpu_tb;

// Inputs

reg [31:0] a, b;

reg [31:0] mem_data_in;

reg [3:0] opcode;

reg load, store;

reg [31:0] address;

reg [4:0] rd;

reg write_enable;

// Outputs

wire [31:0] result;

wire [31:0] mem_data_out;

// Instantiate the FPU

fpu dut (

.a(a),

.b(b),

.mem_data_in(mem_data_in),

.opcode(opcode),

.load(load),

.store(store),

.address(address),

.rd(rd),

.write_enable(write_enable),

.result(result),

.mem_data_out(mem_data_out)

);

// Define special IEEE-754 values

localparam [31:0]

ZERO_POS       = 32'h00000000,    // +0.0

ZERO_NEG       = 32'h80000000,    // -0.0

INF_POS        = 32'h7F800000,    // +Infinity

INF_NEG        = 32'hFF800000,    // -Infinity

NAN_CANONICAL  = 32'h7FC00000,    // Canonical NaN

ONE_POS        = 32'h3F800000,    // +1.0

ONE_NEG        = 32'hBF800000,    // -1.0

TWO_POS        = 32'h40000000,    // +2.0

HALF_POS       = 32'h3F000000,    // +0.5

QUARTER_POS    = 32'h3E800000,    // +0.25

PI             = 32'h40490FDB,    // +3.14159...

E              = 32'h402DF854,    // +2.71828...

SMALL_POS      = 32'h00800000,    // Very small positive number

LARGE_POS      = 32'h7F7FFFFF;    // Very large positive number

// Convert IEEE-754 to real for display purposes

function real ieee_to_real;

input [31:0] ieee;

reg sign;

reg [7:0] exp;

reg [22:0] frac;

real result;

begin

sign = ieee[31];

exp = ieee[30:23];

frac = ieee[22:0];

if (exp == 0) begin

// Denormalized numbers or zero

if (frac == 0)

result = 0.0;

else

result = $itor(frac) / $pow(2, 23) * $pow(2, -126);

end

else if (exp == 255) begin

// Infinity or NaN

if (frac == 0)

result = sign ? -1.0/0.0 : 1.0/0.0;

else

result = 0.0/0.0; // NaN

end

else begin

// Normalized numbers

result = (1.0 + $itor(frac) / $pow(2, 23)) * $pow(2, $signed(exp - 127));

end

if (sign)

result = -result;

ieee_to_real = result;

end

endfunction

// Helper function to display test results

task display_result;

input [31:0] a_val, b_val, result_val;

input [3:0] op;

input pass;

reg [64*8:0] op_str;

begin

case(op)

4'b0000: op_str = "FADD.S";

4'b0001: op_str = "FSUB.S";

4'b0010: op_str = "FMUL.S";

4'b0011: op_str = "FDIV.S";

4'b0100: op_str = "FMIN.S";

4'b0101: op_str = "FMAX.S";

4'b0110: op_str = "FSQRT.S";

4'b1000: op_str = "FEQ.S";

4'b1001: op_str = "FLT.S";

4'b1010: op_str = "FLE.S";

default: op_str = "UNKNOWN";

endcase

$display("Test %s: a = %h (%f), b = %h (%f), result = %h (%f) - %s",

op_str,

a_val, ieee_to_real(a_val),

b_val, ieee_to_real(b_val),

result_val,

(op >= 4'b1000) ? $itor(result_val[0]) : ieee_to_real(result_val),

pass ? "PASS" : "FAIL");

end

endtask

// Test vectors and expected results

reg [31:0] expected_result;

reg test_pass;

// Main test procedure

initial begin

// Initialize

a = 0;

b = 0;

mem_data_in = 0;

opcode = 0;

load = 0;

store = 0;

address = 0;

rd = 0;

write_enable = 0;

$display("\n--- FPU Testbench - Testing All Instructions ---\n");

// Allow some time for initialization

#10;

// ----------------------------------------------------------------

// Test 1: Addition (FADD.S)

// ----------------------------------------------------------------

$display("\nTesting FADD.S Instruction:");

opcode = 4'b0000;

// Test Case 1: 1.0 + 2.0 = 3.0

a = ONE_POS;

b = TWO_POS;

expected_result = 32'h40400000; // 3.0

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: 1.0 + (-1.0) = 0.0

a = ONE_POS;

b = ONE_NEG;

expected_result = ZERO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: 0.0 + 0.0 = 0.0

a = ZERO_POS;

b = ZERO_POS;

expected_result = ZERO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 4: Inf + 1.0 = Inf

a = INF_POS;

b = ONE_POS;

expected_result = INF_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 5: NaN + 1.0 = NaN

a = NAN_CANONICAL;

b = ONE_POS;

#10;

test_pass = ((result[30:23] == 8'hFF) && (result[22] == 1)); // Check if result is NaN

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 2: Subtraction (FSUB.S)

// ----------------------------------------------------------------

$display("\nTesting FSUB.S Instruction:");

opcode = 4'b0001;

// Test Case 1: 2.0 - 1.0 = 1.0

a = TWO_POS;

b = ONE_POS;

expected_result = ONE_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: 1.0 - 1.0 = 0.0

a = ONE_POS;

b = ONE_POS;

expected_result = ZERO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: 0.0 - 1.0 = -1.0

a = ZERO_POS;

b = ONE_POS;

expected_result = ONE_NEG;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 3: Multiplication (FMUL.S)

// ----------------------------------------------------------------

$display("\nTesting FMUL.S Instruction:");

opcode = 4'b0010;

// Test Case 1: 2.0 * 1.0 = 2.0

a = TWO_POS;

b = ONE_POS;

expected_result = TWO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: 0.5 * 0.5 = 0.25

a = HALF_POS;

b = HALF_POS;

expected_result = QUARTER_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: 1.0 * 0.0 = 0.0

a = ONE_POS;

b = ZERO_POS;

expected_result = ZERO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 4: 1.0 * Inf = Inf

a = ONE_POS;

b = INF_POS;

expected_result = INF_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 4: Division (FDIV.S)

// ----------------------------------------------------------------

$display("\nTesting FDIV.S Instruction:");

opcode = 4'b0011;

// Test Case 1: 2.0 / 1.0 = 2.0

a = TWO_POS;

b = ONE_POS;

expected_result = TWO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: 1.0 / 2.0 = 0.5

a = ONE_POS;

b = TWO_POS;

expected_result = HALF_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: 0.0 / 1.0 = 0.0

a = ZERO_POS;

b = ONE_POS;

expected_result = ZERO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 4: 1.0 / 0.0 = Inf

a = ONE_POS;

b = ZERO_POS;

expected_result = INF_POS;

#10;

test_pass = ((result[30:23] == 8'hFF) && (result[22:0] == 0)); // Check if result is Inf

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 5: Minimum (FMIN.S)

// ----------------------------------------------------------------

$display("\nTesting FMIN.S Instruction:");

opcode = 4'b0100;

// Test Case 1: min(1.0, 2.0) = 1.0

a = ONE_POS;

b = TWO_POS;

expected_result = ONE_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: min(-1.0, 1.0) = -1.0

a = ONE_NEG;

b = ONE_POS;

expected_result = ONE_NEG;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: min(NaN, 1.0) = 1.0

a = NAN_CANONICAL;

b = ONE_POS;

expected_result = ONE_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 6: Maximum (FMAX.S)

// ----------------------------------------------------------------

$display("\nTesting FMAX.S Instruction:");

opcode = 4'b0101;

// Test Case 1: max(1.0, 2.0) = 2.0

a = ONE_POS;

b = TWO_POS;

expected_result = TWO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: max(-1.0, 1.0) = 1.0

a = ONE_NEG;

b = ONE_POS;

expected_result = ONE_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: max(NaN, 1.0) = 1.0

a = NAN_CANONICAL;

b = ONE_POS;

expected_result = ONE_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 7: Square Root (FSQRT.S)

// ----------------------------------------------------------------

$display("\nTesting FSQRT.S Instruction:");

opcode = 4'b0110;

// Test Case 1: sqrt(4.0) = 2.0

a = 32'h40800000; // 4.0

expected_result = TWO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: sqrt(1.0) = 1.0

a = ONE_POS;

expected_result = ONE_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: sqrt(0.0) = 0.0

a = ZERO_POS;

expected_result = ZERO_POS;

#10;

test_pass = (result == expected_result);

display_result(a, b, result, opcode, test_pass);

// Test Case 4: sqrt(-1.0) = NaN

a = ONE_NEG;

#10;

test_pass = ((result[30:23] == 8'hFF) && (result[22] == 1)); // Check if result is NaN

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 8: Equal (FEQ.S)

// ----------------------------------------------------------------

$display("\nTesting FEQ.S Instruction:");

opcode = 4'b1000;

// Test Case 1: 1.0 == 1.0 -> true (1)

a = ONE_POS;

b = ONE_POS;

expected_result = 32'h00000001;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: 1.0 == 2.0 -> false (0)

a = ONE_POS;

b = TWO_POS;

expected_result = 32'h00000000;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: +0.0 == -0.0 -> true (1)

a = ZERO_POS;

b = ZERO_NEG;

expected_result = 32'h00000001;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 4: NaN == 1.0 -> false (0)

a = NAN_CANONICAL;

b = ONE_POS;

expected_result = 32'h00000000;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 9: Less Than (FLT.S)

// ----------------------------------------------------------------

$display("\nTesting FLT.S Instruction:");

opcode = 4'b1001;

// Test Case 1: 1.0 < 2.0 -> true (1)

a = ONE_POS;

b = TWO_POS;

expected_result = 32'h00000001;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: 2.0 < 1.0 -> false (0)

a = TWO_POS;

b = ONE_POS;

expected_result = 32'h00000000;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: -1.0 < 1.0 -> true (1)

a = ONE_NEG;

b = ONE_POS;

expected_result = 32'h00000001;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 4: NaN < 1.0 -> false (0)

a = NAN_CANONICAL;

b = ONE_POS;

expected_result = 32'h00000000;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 10: Less Than or Equal (FLE.S)

// ----------------------------------------------------------------

$display("\nTesting FLE.S Instruction:");

opcode = 4'b1010;

// Test Case 1: 1.0 <= 2.0 -> true (1)

a = ONE_POS;

b = TWO_POS;

expected_result = 32'h00000001;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 2: 1.0 <= 1.0 -> true (1)

a = ONE_POS;

b = ONE_POS;

expected_result = 32'h00000001;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 3: 2.0 <= 1.0 -> false (0)

a = TWO_POS;

b = ONE_POS;

expected_result = 32'h00000000;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// Test Case 4: -1.0 <= 1.0 -> true (1)

a = ONE_NEG;

b = ONE_POS;

expected_result = 32'h00000001;

#10;

test_pass = (result[0] == expected_result[0]);

display_result(a, b, result, opcode, test_pass);

// ----------------------------------------------------------------

// Test 11: Load and Store Operations

// ----------------------------------------------------------------

$display("\nTesting Load/Store Instructions:");

// Test Load Operation (FLW)

opcode = 4'b0000;  // Doesn't matter for load/store

load = 1;

store = 0;

address = 32'h00000004;  // Aligned address

mem_data_in = PI;  // Load value of PI

#10;

test_pass = (result == PI);

$display("Test FLW: mem[%h] = %h (%f), result = %h (%f) - %s",

address, mem_data_in, ieee_to_real(mem_data_in),

result, ieee_to_real(result),

test_pass ? "PASS" : "FAIL");

// Test Load Operation with misaligned address

address = 32'h00000001;  // Misaligned address

#10;

$display("Test FLW (misaligned): address = %h, fcsr[3] should be set", address);

// Test Store Operation (FSW)

load = 0;

store = 1;

address = 32'h00000008;  // Aligned address

a = E;  // Store value of E

#10;

test_pass = (mem_data_out == E);

$display("Test FSW: a = %h (%f), mem_data_out = %h (%f) - %s",

a, ieee_to_real(a),

mem_data_out, ieee_to_real(mem_data_out),

test_pass ? "PASS" : "FAIL");

// Test Store Operation with misaligned address

address = 32'h00000002;  // Misaligned address

#10;

$display("Test FSW (misaligned): address = %h, fcsr[3] should be set", address);

// Reset load/store signals

load = 0;

store = 0;

// Test Register Write

opcode = 4'b0000;  // ADD

a = ONE_POS;

b = TWO_POS;

rd = 5'd10;  // f10 register

write_enable = 0;

#10;

// Trigger register write

write_enable = 1;

#10;

write_enable = 0;

$display("Test Register Write: Writing %h to f%0d", result, rd);

// End simulation

$display("\n--- All FPU Instructions Tested ---\n");

#10 $finish;

end

endmodule

`timescale 1ns / 1ps

module fpu_tb;

// Testbench signals

reg [31:0] a, b;

reg [31:0] mem_data_in;

reg [3:0] opcode;

reg load, store;

reg [31:0] address;

reg [4:0] rd;

reg write_enable;

wire [31:0] result;

wire [31:0] mem_data_out;

// Instantiate the FPU module

fpu uut (

.a(a),

.b(b),

.mem_data_in(mem_data_in),

.opcode(opcode),

.load(load),

.store(store),

.address(address),

.rd(rd),

.write_enable(write_enable),

.result(result),

.mem_data_out(mem_data_out)

);

// Dump waveforms

initial begin

$dumpfile("fpu_tb.vcd");

$dumpvars(0, fpu_tb);

end

// Helper tasks for floating-point operations

task test_binary_op;

input [3:0] op;

input [31:0] operand_a;

input [31:0] operand_b;

input [4:0] dest_reg;

input [31:0] expected_result;

begin

a = operand_a;

b = operand_b;

opcode = op;

rd = dest_reg;

load = 0;

store = 0;

address = 0;

write_enable = 0;

#5;

write_enable = 1;

#5;

write_enable = 0;

$display("Op: %b, A: %h, B: %h, Result: %h, Expected: %h, %s",

op, operand_a, operand_b, result, expected_result,

(result === expected_result) ? "PASS" : "FAIL");

#10;

end

endtask

task test_unary_op;

input [3:0] op;

input [31:0] operand_a;

input [4:0] dest_reg;

input [31:0] expected_result;

begin

a = operand_a;

b = 32'h0;

opcode = op;

rd = dest_reg;

load = 0;

store = 0;

address = 0;

write_enable = 0;

#5;

write_enable = 1;

#5;

write_enable = 0;

$display("Op: %b, A: %h, Result: %h, Expected: %h, %s",

op, operand_a, result, expected_result,

(result === expected_result) ? "PASS" : "FAIL");

#10;

end

endtask

task test_load_store;

input is_load;

input [31:0] operand_a;

input [31:0] mem_address;

input [31:0] memory_data;

input [4:0] dest_reg;

input [31:0] expected_result;

begin

a = operand_a;

address = mem_address;

mem_data_in = memory_data;

rd = dest_reg;

load = is_load;

store = !is_load;

write_enable = 0;

#5;

if (is_load) begin

write_enable = 1;

#5;

write_enable = 0;

$display("Load: Addr: %h, Data: %h, Result: %h, Expected: %h, %s",

mem_address, memory_data, result, expected_result,

(result === expected_result) ? "PASS" : "FAIL");

end else begin

#5;

$display("Store: Addr: %h, Data Out: %h, Expected: %h, %s",

mem_address, mem_data_out, expected_result,

(mem_data_out === expected_result) ? "PASS" : "FAIL");

end

#10;

end

endtask

// Main test sequence

initial begin

$display("Starting FPU Testbench");

// Initialize

a = 0;

b = 0;

mem_data_in = 0;

opcode = 0;

load = 0;

store = 0;

address = 0;

rd = 0;

write_enable = 0;

#10;

$display("\n=== Testing FADD.S (ADD) ===");

// 3.5 + 2.25 = 5.75

test_binary_op(4'b0000, 32'h40600000, 32'h40100000, 5'd1, 32'h40B80000);

// -2.5 + 1.5 = -1.0

test_binary_op(4'b0000, 32'hC0200000, 32'h3FC00000, 5'd2, 32'hBF800000);

$display("\n=== Testing FSUB.S (SUB) ===");

// 3.5 - 2.25 = 1.25

test_binary_op(4'b0001, 32'h40600000, 32'h40100000, 5'd3, 32'h3FA00000);

// -2.5 - 1.5 = -4.0

test_binary_op(4'b0001, 32'hC0200000, 32'h3FC00000, 5'd4, 32'hC0800000);

$display("\n=== Testing FMUL.S (MUL) ===");

// 3.5 * 2.0 = 7.0

test_binary_op(4'b0010, 32'h40600000, 32'h40000000, 5'd5, 32'h40E00000);

// -2.5 * 2.0 = -5.0

test_binary_op(4'b0010, 32'hC0200000, 32'h40000000, 5'd6, 32'hC0A00000);

$display("\n=== Testing FDIV.S (DIV) ===");

// 3.5 / 2.0 = 1.75

test_binary_op(4'b0011, 32'h40600000, 32'h40000000, 5'd7, 32'h3FE00000);

// -8.0 / 2.0 = -4.0

test_binary_op(4'b0011, 32'hC1000000, 32'h40000000, 5'd8, 32'hC0800000);

$display("\n=== Testing FMIN.S ===");

// min(3.5, 2.0) = 2.0

test_binary_op(4'b0100, 32'h40600000, 32'h40000000, 5'd9, 32'h40000000);

// min(-2.5, 1.5) = -2.5

test_binary_op(4'b0100, 32'hC0200000, 32'h3FC00000, 5'd10, 32'hC0200000);

$display("\n=== Testing FMAX.S ===");

// max(3.5, 2.0) = 3.5

test_binary_op(4'b0101, 32'h40600000, 32'h40000000, 5'd11, 32'h40600000);

// max(-2.5, 1.5) = 1.5

test_binary_op(4'b0101, 32'hC0200000, 32'h3FC00000, 5'd12, 32'h3FC00000);

$display("\n=== Testing FSQRT.S ===");

// sqrt(4.0) = 2.0

test_unary_op(4'b0110, 32'h40800000, 5'd13, 32'h40000000);

// sqrt(9.0) = 3.0

test_unary_op(4'b0110, 32'h41100000, 5'd14, 32'h40400000);

$display("\n=== Testing FEQ.S ===");

// 3.5 == 3.5 ? 1 : 0 (true)

test_binary_op(4'b1000, 32'h40600000, 32'h40600000, 5'd15, 32'h00000001);

// 3.5 == 2.0 ? 1 : 0 (false)

test_binary_op(4'b1000, 32'h40600000, 32'h40000000, 5'd16, 32'h00000000);

$display("\n=== Testing FLT.S ===");

// 2.0 < 3.5 ? 1 : 0 (true)

test_binary_op(4'b1001, 32'h40000000, 32'h40600000, 5'd17, 32'h00000001);

// 3.5 < 2.0 ? 1 : 0 (false)

test_binary_op(4'b1001, 32'h40600000, 32'h40000000, 5'd18, 32'h00000000);

$display("\n=== Testing FLE.S ===");

// 2.0 <= 3.5 ? 1 : 0 (true)

test_binary_op(4'b1010, 32'h40000000, 32'h40600000, 5'd19, 32'h00000001);

// 3.5 <= 3.5 ? 1 : 0 (true)

test_binary_op(4'b1010, 32'h40600000, 32'h40600000, 5'd20, 32'h00000001);

$display("\n=== Testing FCVT.W.S (float to signed int) ===");

// Convert 3.75 to int = 3

test_unary_op(4'b1100, 32'h40700000, 5'd21, 32'h00000003);

// Convert -2.25 to int = -2

test_unary_op(4'b1100, 32'hC0100000, 5'd22, 32'hFFFFFFFE);

$display("\n=== Testing FCVT.WU.S (float to unsigned int) ===");

// Convert 3.75 to uint = 3

test_unary_op(4'b1101, 32'h40700000, 5'd23, 32'h00000003);

// Convert -2.25 to uint = 0 (negative becomes 0)

test_unary_op(4'b1101, 32'hC0100000, 5'd24, 32'h00000000);

$display("\n=== Testing FCVT.S.W (signed int to float) ===");

// Convert int 42 to float = 42.0

test_unary_op(4'b1110, 32'h0000002A, 5'd25, 32'h42280000);

// Convert int -15 to float = -15.0

test_unary_op(4'b1110, 32'hFFFFFFF1, 5'd26, 32'hC1700000);

$display("\n=== Testing FCVT.S.WU (unsigned int to float) ===");

// Convert uint 42 to float = 42.0

test_unary_op(4'b1111, 32'h0000002A, 5'd27, 32'h42280000);

// Convert uint 3000000000 (large value) to float

test_unary_op(4'b1111, 32'hB2D05E00, 5'd28, 32'h4E32D05E);

$display("\n=== Testing FLW (Load Float) ===");

// Load float 3.5 from memory

test_load_store(1, 32'h00000000, 32'h00000100, 32'h40600000, 5'd29, 32'h40600000);

$display("\n=== Testing FSW (Store Float) ===");

// Store float -2.5 to memory

test_load_store(0, 32'hC0200000, 32'h00000200, 32'h00000000, 5'd0, 32'hC0200000);

$display("\nFPU Testbench Completed");

#50 $finish;

end

endmodule